Wireless Standards and Configuration

Setup, Channels, Security, MAC Filtering and ACLs

# 1. Theory Summary

- SSID: the network name that the router broadcasts.

- Frequency bands: 2.4 GHz (longer range, more interference, only channels 1, 6 and 11 do not overlap) and 5 GHz (faster, shorter range, many non-overlapping channels).

- Channel selection: neighbouring networks on the same or overlapping channels cause interference. Pick the channel with the fewest networks and least overlap.

- Security: WPA3 is the strongest, WPA2 with AES (CCMP) is the minimum recommended. Avoid WEP, WPA and TKIP, and turn off WPS.

- MAC filtering: allows only listed device hardware addresses. It is a basic access control, but MAC addresses can be spoofed, so it must not replace strong encryption.

- ACL: a list of permit or deny rules that filters traffic by IP address, protocol or port. Domain names are not matched directly by classic ACLs.

# 2. Task 1: Basic Wireless Setup

Steps:

- Power on the router and open its admin page in a browser (commonly 192.168.0.1 or 192.168.1.1; check the label under the router).

- Log in and open the Wireless settings. Note the SSID and channel.

- Connect your laptop or phone to the SSID and confirm internet access.

| Item | Your observation |
| --- | --- |
| Router model |  |
| SSID |  |
| Band (2.4 GHz or 5 GHz) |  |
| Default channel / mode | (for example Auto, channel 6) |
| Default security | (for example WPA2-PSK) |
| Connected device | (laptop or phone) |

# 3. Task 2: Select a Manual Channel and Check Interference

Steps:

- Install a Wi-Fi analyzer app (for example WiFiman, NetSpot or Wifi Analyzer on Android).

- Open the channel graph on the 2.4 GHz band and note how many networks sit on channels 1, 6 and 11 and how strongly they overlap.

- In the router's wireless settings, change Channel from Auto to the least crowded of 1, 6 or 11, and save.

- Reconnect, rescan with the app, and compare the graph before and after.

| Channel | Networks seen (before) | Networks seen (after) |
| --- | --- | --- |
| 1 |  |  |
| 6 |  |  |
| 11 |  |  |

Tip: channels 1, 6 and 11 are 25 MHz apart, so they do not overlap each other. A network on channel 3, for example, overlaps both 1 and 6. If the 5 GHz band is available and your devices support it, it usually has far less interference.

# 4. Task 3: Enable WPA2 or WPA3 Security

Steps:

- Open Wireless Security settings and select WPA3 (or WPA2-PSK with AES if WPA3 is not available, or WPA2/WPA3 mixed mode for older devices).

- Set a strong passphrase: at least 12 characters, mixing upper and lower case, numbers and symbols, and not related to personal details.

- Also change the router admin password and disable WPS.

- Forget the old network on your device and reconnect using the new password. Try a wrong password once to confirm that it is rejected.

| Check | Result |
| --- | --- |
| Security mode set |  |
| Connected with the new password? |  |
| Wrong password rejected? |  |

# 5. Task 4: MAC Filtering

Finding the MAC address:

- Android: Settings > About phone > Status > Wi-Fi MAC address.

- iPhone: Settings > Wi-Fi > the (i) next to your network > Private Wi-Fi Address.

- Windows: open Command Prompt and run ipconfig /all, then look for Physical Address of the Wi-Fi adapter.

Note: modern phones use a random (private) MAC address per network. Either turn off the private or random MAC for this network, or add the address shown for this network to the list.

Steps:

- In the router, open Wireless > MAC Filtering (or Access Control).

- Set the mode to Allow only the listed devices and add your device's MAC address.

- Connect with the listed device (should succeed), then try a second device that is not on the list (should fail).

| Device | MAC address | Connection result |
| --- | --- | --- |
| Allowed device |  |  |
| Other device |  |  |

Observation: the unlisted device cannot connect even with the correct Wi-Fi password. Limitation: an attacker can copy a listed MAC address, so MAC filtering is not strong security by itself.

# 6. Task 5: ACL to Block a Website

Option A: home router. Most home routers do not let you write an ACL directly. Use Access Restrictions, Parental Controls or Website Blocking, add www.youtube.com (and youtube.com) as a blocked domain, and apply it to all devices.

Option B: Cisco router (lab or Packet Tracer). A standard IOS ACL matches IP addresses, not names, so first find the website's address:

C:\> nslookup www.youtube.com

Then block it on the router interface facing the Wi-Fi/LAN network (replace 203.0.113.10 with the address returned, and repeat for each address):

Router(config)# ip access-list extended BLOCK-YOUTUBE

Router(config-ext-nacl)# deny ip 192.168.1.0 0.0.0.255 host 203.0.113.10

Router(config-ext-nacl)# permit ip any any

Router(config-ext-nacl)# exit

Router(config)# interface gigabitEthernet 0/1

Router(config-if)# ip access-group BLOCK-YOUTUBE in

Router(config-if)# end

Router# show access-lists

Notes:

- The final permit ip any any is needed, because every ACL ends with an implicit deny all.

- The in direction on the LAN-facing interface filters traffic as it enters from the Wi-Fi network.

- Large sites use many changing IP addresses and also QUIC (UDP 443). Domain-based website blocking, or a DNS filter, is more reliable for real use.

Testing:

- From a connected device, open www.youtube.com in a browser. It should not load (timeout or blocked page).

- Open another site such as www.google.com. It should still load, which shows the ACL is blocking only the target.

- If the site still opens, clear the browser cache and run ipconfig /flushdns, then check show access-lists for hit counts.

| Test | Result |
| --- | --- |
| www.youtube.com from a Wi-Fi device |  |
| Another website (for example google.com) |  |

# 7. Troubleshooting

| Problem | Check |
| --- | --- |
| Cannot connect after changing the channel | Rescan Wi-Fi; some old devices do not support channels 12 to 13 |
| Cannot connect after changing security | Forget the network on the device and reconnect |
| Allowed device is blocked by MAC filtering | Use the correct (not randomized) MAC address |
| Website still opens after the block | Flush DNS, clear cache, check all resolved IPs and the ACL direction |

# 8. Conclusion

A basic wireless network was set up, the channel was changed manually to reduce interference, WPA2 or WPA3 was enabled with a strong password, MAC filtering restricted access to a chosen device, and an ACL or website block was used to stop access to a specific website. Layering these controls gives better protection than any single one.

# 9. Submission Checklist

- Screenshot of router wireless settings (SSID, default channel)

- Analyzer screenshots before and after the channel change

- Screenshot of the security mode (WPA2 or WPA3)

- MAC filter list and the result for the unlisted device

- ACL or website block settings and the blocked-site test result