# DAY 1: STATIC ROUTING CONCEPTS

Theory + Cisco Packet Tracer Practical | Ahmedabad–Surat Branch Network

# Theory

What is Static Routing? Static routing is a routing method in which a network administrator manually configures the path to a destination network. The router does not automatically learn or change the route using a dynamic routing protocol.

## Advantages and Disadvantages

| Type | Point | Real-world analogy |
| --- | --- | --- |
| Advantage | Simple and predictable for small, stable networks. | A small Swiggy branch with fixed locations can use a known path. |
| Advantage | Low routing-protocol overhead because routers do not exchange route updates. | A fixed branch path avoids continuous routing-update traffic. |
| Disadvantage | Does not automatically adapt when a configured path fails. | If the Ahmedabad–Surat path fails, a manually configured route needs intervention or a configured backup. |
| Disadvantage | Manual configuration becomes difficult as the network grows. | An IRCTC-scale network with many locations would require many routes to be maintained manually. |

# Practical: Two-Router Static Routing

Goal: Configure two routers connected by a serial link so that an Ahmedabad LAN and a Surat LAN can communicate using static routes.

## 1. Topology and IP Addressing

PC1 — Switch1 — R1 (Ahmedabad) ===== Serial ===== R2 (Surat) — Switch2 — PC2

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | G0/0 | 192.168.1.1 | 255.255.255.0 |
| R1 | S0/0/0 | 10.0.0.1 | 255.255.255.252 |
| R2 | S0/0/0 | 10.0.0.2 | 255.255.255.252 |
| R2 | G0/0 | 192.168.2.1 | 255.255.255.0 |
| PC1 | NIC | 192.168.1.10 | 255.255.255.0 |
| PC2 | NIC | 192.168.2.10 | 255.255.255.0 |

PC1 Default Gateway: 192.168.1.1    |    PC2 Default Gateway: 192.168.2.1

## 2. Router1 (Ahmedabad) - Complete Commands

enable configure terminal hostname Ahmedabad interface gigabitEthernet 0/0 ip address 192.168.1.1 255.255.255.0 no shutdown exit interface serial 0/0/0 ip address 10.0.0.1 255.255.255.252 clock rate 64000 no shutdown exit ip route 192.168.2.0 255.255.255.0 10.0.0.2 end write memory

## 3. Router2 (Surat) - Complete Commands

enable configure terminal hostname Surat interface gigabitEthernet 0/0 ip address 192.168.2.1 255.255.255.0 no shutdown exit interface serial 0/0/0 ip address 10.0.0.2 255.255.255.252 no shutdown exit ip route 192.168.1.0 255.255.255.0 10.0.0.1 end write memory

Note: Configure 'clock rate 64000' only on the DCE side of the serial connection. Your Packet Tracer router may use different interface names such as Fa0/0 or S0/3/0; verify with 'show ip interface brief'.

## 4. PC Configuration

| PC | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC1 - Ahmedabad | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC2 - Surat | 192.168.2.10 | 255.255.255.0 | 192.168.2.1 |

## 5. Testing and Verification

From Router1, test the PC connected to Router2:

Ahmedabad# ping 192.168.2.10 Ahmedabad# show ip route Ahmedabad# show ip interface brief

From Ahmedabad PC1:

ping 192.168.2.10

From Surat PC2:

ping 192.168.1.10

If the configuration is correct and all interfaces are up, the PCs in both city LANs should communicate successfully.

# Task-wise Answers

Task 1: Two routers are connected through a serial cable. R1 uses 10.0.0.1/30 and R2 uses 10.0.0.2/30 on the serial link. Each router also has a separate /24 LAN.

Task 2: Router1 uses the static route 'ip route 192.168.2.0 255.255.255.0 10.0.0.2'. Ping 192.168.2.10 from Router1 to verify reachability.

Task 3: R1 represents Ahmedabad and R2 represents Surat. Static routes are configured in both directions so 192.168.1.0/24 and 192.168.2.0/24 can communicate.

Task 4: Static routing is simple, predictable, and low-overhead for small networks, but it does not automatically adapt to failures and becomes difficult to maintain in large networks.