Inter-VLAN Routing Basics

Cisco Packet Tracer Lab Report

# 1. Theory: Methods of Inter-VLAN Routing

Devices in different VLANs are in different broadcast domains and different IP subnets, so they need a Layer 3 device to communicate.

- Router-on-a-stick: one physical router interface is connected to a switch trunk port and split into logical sub-interfaces, one per VLAN. Each sub-interface is the default gateway for its VLAN.

- Layer 3 switch: the switch routes between VLANs itself using SVIs (Switch Virtual Interfaces, for example interface vlan 10) with ip routing enabled.

- Legacy method: one router interface per VLAN. It does not scale and is rarely used.

# 2. Task 1: Topology and VLANs

Devices: 1 router (2911), 1 switch (2960), 3 PCs. Switch Fa0/1 connects to router Gi0/0 and is a trunk.

| VLAN | Name (Service) | Switch Port | PC IP Address | Gateway |
| --- | --- | --- | --- | --- |
| 10 | Foodies (Zomato) | Fa0/2 (PC1) | 192.168.10.10/24 | 192.168.10.1 |
| 20 | Shoppers (Flipkart) | Fa0/3 (PC2) | 192.168.20.10/24 | 192.168.20.1 |
| 30 | Streamers (Spotify) | Fa0/4 (PC3) | 192.168.30.10/24 | 192.168.30.1 |

Switch configuration:

Switch> enable

Switch# configure terminal

Switch(config)# vlan 10

Switch(config-vlan)# name Foodies

Switch(config-vlan)# vlan 20

Switch(config-vlan)# name Shoppers

Switch(config-vlan)# vlan 30

Switch(config-vlan)# name Streamers

Switch(config-vlan)# exit

Switch(config)# interface fastEthernet 0/2

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config-if)# interface fastEthernet 0/3

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 20

Switch(config-if)# interface fastEthernet 0/4

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 30

Switch(config-if)# interface fastEthernet 0/1

Switch(config-if)# switchport mode trunk

Switch(config-if)# end

Switch# copy running-config startup-config

# 3. Task 2: Router-on-a-Stick Configuration

The physical interface has no IP address and must be turned on. Each sub-interface needs the VLAN tag and the gateway IP:

Router> enable

Router# configure terminal

Router(config)# interface gigabitEthernet 0/0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface gigabitEthernet 0/0.10

Router(config-subif)# encapsulation dot1Q 10

Router(config-subif)# ip address 192.168.10.1 255.255.255.0

Router(config-subif)# exit

Router(config)# interface gigabitEthernet 0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 192.168.20.1 255.255.255.0

Router(config-subif)# exit

Router(config)# interface gigabitEthernet 0/0.30

Router(config-subif)# encapsulation dot1Q 30

Router(config-subif)# ip address 192.168.30.1 255.255.255.0

Router(config-subif)# end

Router# copy running-config startup-config

Verify with show ip interface brief, show ip route (three connected C routes), and show interfaces trunk on the switch.

# 4. Task 3: PC Addressing and Test

Set each PC's IP address, subnet mask 255.255.255.0 and default gateway as in the table in section 2. Then ping the gateway first, then the other VLANs:

| Ping | Expected | Note |
| --- | --- | --- |
| PC1 to 192.168.10.1 | Success | Own gateway |
| PC1 to PC2 (Foodies to Shoppers) | Success | Routed by the sub-interfaces |
| PC2 to PC3 (Shoppers to Streamers) | Success | Routed by the sub-interfaces |
| PC1 to PC3 (Foodies to Streamers) | Success | Routed by the sub-interfaces |

The first ping may time out while ARP completes. Inter-VLAN traffic goes up the trunk to the router and back down the same link, which is why it is called a 'stick'.

# 5. Task 4: Troubleshooting Scenario

Scenario: Shoppers (PC2) cannot ping Streamers (PC3), although Foodies can still reach Shoppers.

Step 1: Isolate the problem.

- Ping from PC3 to its own gateway 192.168.30.1. It fails, so the problem is between PC3 and the router, not in the Shoppers VLAN.

- PC1 to PC2 works, which shows the trunk and the Foodies and Shoppers sub-interfaces are fine.

Step 2: Check the configuration.

Router# show ip interface brief

Router# show running-config | section Gi0/0.30

Switch# show vlan brief

Switch# show interfaces trunk

Fault found in this scenario: the sub-interface has the wrong VLAN tag.

interface GigabitEthernet0/0.30

encapsulation dot1Q 300

ip address 192.168.30.1 255.255.255.0

The router listens for VLAN 300, but the PC3 port is in VLAN 30, so the tagged frames are never matched.

Step 3: Fix and verify.

Router(config)# interface gigabitEthernet 0/0.30

Router(config-subif)# encapsulation dot1Q 30

Router(config-subif)# end

Ping PC3 to its gateway, then PC2 to PC3. Both now succeed.

Other faults to check with the hint (VLAN assignments, sub-interfaces, trunk):

| Fault | Symptom | Fix |
| --- | --- | --- |
| Access port in the wrong VLAN | That PC cannot reach its gateway | switchport access vlan 30 on Fa0/4 |
| Switch link to router is access, not trunk | No inter-VLAN traffic at all | switchport mode trunk on Fa0/1 |
| VLAN missing from trunk allowed list | One VLAN cannot reach the router | switchport trunk allowed vlan add 30 |
| Wrong sub-interface IP or mask | Gateway unreachable or wrong subnet | Correct the ip address command |
| Wrong default gateway on the PC | Local pings work, other VLANs fail | Set the gateway to the sub-interface IP |
| Physical interface shut down | All sub-interfaces down | no shutdown on Gi0/0 |
| VLAN not created on the switch | Ports show in no active VLAN | Create it with the vlan command |

# 6. Task 5: Router-on-a-Stick vs Layer 3 Switch

A Layer 3 switch routes with SVIs. Example (Packet Tracer 3560):

Switch(config)# ip routing

Switch(config)# interface vlan 10

Switch(config-if)# ip address 192.168.10.1 255.255.255.0

Switch(config-if)# no shutdown

| Method | Advantages | Disadvantages |
| --- | --- | --- |
| Router-on-a-stick | 1. Low cost: uses a router and a single link you may already own. 2. Simple to set up and understand for small networks. | 1. The single trunk link is a bandwidth bottleneck and single point of failure. 2. Software routing on the router is slower and does not scale well as VLANs and traffic grow. |
| Layer 3 switch | 1. Fast hardware routing at wire speed, with no external link to congest. 2. Scales easily: add an SVI per VLAN and keep routing inside the switch. | 1. Higher cost than a basic switch and router pair. 2. More complex to configure and fewer WAN features than a router. |

# 7. Conclusion

Router-on-a-stick lets VLANs communicate through one trunk and sub-interfaces. Troubleshooting works best step by step: test the gateway, then check the VLAN assignment, trunk and sub-interface settings. For larger networks, a Layer 3 switch is the better choice.