VLAN Trunking and Access Ports

Cisco Packet Tracer Lab Report

# 1. Theory: Trunking Protocols

- Access port: carries traffic for one VLAN only. Frames are untagged and the end device (PC) is unaware of VLANs.

- Trunk port: carries traffic for many VLANs between switches by adding a VLAN tag to each frame.

- IEEE 802.1Q: open standard, inserts a 4-byte tag into the Ethernet frame. Untagged traffic belongs to the native VLAN (default VLAN 1).

- ISL (Inter-Switch Link): Cisco proprietary, encapsulates the whole frame with a 26-byte header and 4-byte trailer. It is deprecated and not supported on newer switches.

- Both ends of a trunk must use the same encapsulation, or the link will not pass VLAN traffic.

Note for Packet Tracer: the 2960 switch supports only 802.1Q and rejects the 'switchport trunk encapsulation' command. Use the multilayer Cisco 3560-24PS for this lab so that both dot1q and ISL can be configured.

# 2. Topology and Addressing

Two switches (Switch1, Switch2) are joined with a crossover cable on FastEthernet0/1. Three PCs connect to Switch1. Two extra PCs on Switch2 are used to test traffic across the trunk.

| Device | Switch Port | VLAN | IP Address | Mask |
| --- | --- | --- | --- | --- |
| PC1 | Switch1 Fa0/2 | 10 | 192.168.10.10 | 255.255.255.0 |
| PC2 | Switch1 Fa0/3 | 20 | 192.168.20.10 | 255.255.255.0 |
| PC3 | Switch1 Fa0/4 | 30 | 192.168.30.10 | 255.255.255.0 |
| PC4 | Switch2 Fa0/2 | 10 | 192.168.10.20 | 255.255.255.0 |
| PC5 | Switch2 Fa0/5 | 20 | 192.168.20.20 | 255.255.255.0 |

# 3. Task 1: Configure the Trunk (802.1Q)

Run on both switches (change the hostname on Switch2):

Switch> enable

Switch# configure terminal

Switch(config)# hostname Switch1

Switch1(config)# interface fastEthernet 0/1

Switch1(config-if)# switchport trunk encapsulation dot1q

Switch1(config-if)# switchport mode trunk

Switch1(config-if)# end

Verify with: show interfaces trunk

# 4. Task 2: Create VLANs and Assign Access Ports

Create the VLANs on both switches:

Switch1(config)# vlan 10

Switch1(config-vlan)# name VLAN10

Switch1(config-vlan)# vlan 20

Switch1(config-vlan)# name VLAN20

Switch1(config-vlan)# vlan 30

Switch1(config-vlan)# name VLAN30

Switch1(config-vlan)# exit

Assign the access ports on Switch1:

Switch1(config)# interface fastEthernet 0/2

Switch1(config-if)# switchport mode access

Switch1(config-if)# switchport access vlan 10

Switch1(config-if)# interface fastEthernet 0/3

Switch1(config-if)# switchport mode access

Switch1(config-if)# switchport access vlan 20

Switch1(config-if)# interface fastEthernet 0/4

Switch1(config-if)# switchport mode access

Switch1(config-if)# switchport access vlan 30

Switch1(config-if)# end

On Switch2, put Fa0/2 in VLAN 10 and Fa0/5 in VLAN 20 as access ports in the same way.

Verify with: show vlan brief

# 5. Task 3: Test Connectivity

| Test (ping from, to) | Expected | Reason |
| --- | --- | --- |
| PC1 to PC2 (VLAN 10 to 20) | Fails | Different VLANs, no router |
| PC1 to PC3 (VLAN 10 to 30) | Fails | Different VLANs, no router |
| PC2 to PC3 (VLAN 20 to 30) | Fails | Different VLANs, no router |
| PC1 to PC4 (VLAN 10, across trunk) | Succeeds | Same VLAN, trunk carries VLAN 10 |
| PC2 to PC5 (VLAN 20, across trunk) | Succeeds | Same VLAN, trunk carries VLAN 20 |

Because all three PCs on Switch1 are in different VLANs, same-VLAN communication is shown with the extra PCs on Switch2 across the trunk. Allow the first ping a few seconds (ARP) before judging results. Communication between different VLANs would need a router (router-on-a-stick) or a Layer 3 switch.

# 6. Task 4: Change the Trunk to ISL

On both switches, set the encapsulation before the mode:

Switch1(config)# interface fastEthernet 0/1

Switch1(config-if)# switchport trunk encapsulation isl

Switch1(config-if)# switchport mode trunk

Switch1(config-if)# end

Switch1# show interfaces trunk

Results to record:

- Both ends set to ISL: the trunk comes up (encapsulation shows isl) and PC1 to PC4 still succeeds. VLAN traffic passes.

- Only one end set to ISL (the other still dot1q): the encapsulations do not match, the trunk fails to pass VLAN traffic, and PC1 to PC4 fails. Fix by matching both ends.

- If the switch rejects the command, it is a 2960, which supports only 802.1Q. Replace it with a 3560.

# 7. Task 5: Fa0/5 Accidentally Set as a Trunk

The fault on Switch2:

Switch2(config)# interface fastEthernet 0/5

Switch2(config-if)# switchport trunk encapsulation dot1q

Switch2(config-if)# switchport mode trunk

Impact on PC5 (192.168.20.20), which was in VLAN 20:

- A trunk port expects tagged frames and carries all VLANs. A PC sends untagged frames, so the switch places them in the native VLAN (VLAN 1), not VLAN 20.

- PC5 can no longer reach PC2 or any other VLAN 20 device. Pings fail even though the link light is green.

- PC5 would also receive traffic for VLAN 1 and, with an unrestricted trunk, is exposed to frames from all VLANs. This is a security risk (VLAN hopping).

- If PC5 used DHCP, it would not get an address from the VLAN 20 pool.

Fix:

Switch2(config)# interface fastEthernet 0/5

Switch2(config-if)# switchport mode access

Switch2(config-if)# switchport access vlan 20

After the fix, PC5 to PC2 succeeds again.

# 8. Conclusion

Trunk ports carry multiple VLANs between switches using 802.1Q (or ISL), while access ports serve a single VLAN for end devices. Both trunk ends must use the same encapsulation, and a PC port wrongly set as a trunk loses its VLAN membership and connectivity.