Weekly Review: VLAN and Inter-VLAN Routing

Cisco Packet Tracer Practical Report

# 1. Objective

Build a three-switch network with two VLANs (Sales and Support), trunk the switches together, and enable communication between the VLANs using router-on-a-stick.

# 2. Topology and Addressing

Devices: 1 router (2911), 3 switches (2960: SW1, SW2, SW3), 6 PCs. SW1 connects to the router and to SW2 and SW3. Each switch has one Sales PC and one Support PC.

Router R1

| Gi0/0

| Gi0/1

SW2 ----- SW1 ----- SW3

(Fa0/1 to SW1 Fa0/1)   (Fa0/1 to SW1 Fa0/2)

| PC | Switch / Port | VLAN | IP Address | Mask | Gateway |
| --- | --- | --- | --- | --- | --- |
| PC1 | SW1 Fa0/3 | 10 Sales | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 |
| PC2 | SW1 Fa0/4 | 20 Support | 192.168.20.11 | 255.255.255.0 | 192.168.20.1 |
| PC3 | SW2 Fa0/2 | 10 Sales | 192.168.10.12 | 255.255.255.0 | 192.168.10.1 |
| PC4 | SW2 Fa0/3 | 20 Support | 192.168.20.12 | 255.255.255.0 | 192.168.20.1 |
| PC5 | SW3 Fa0/2 | 10 Sales | 192.168.10.13 | 255.255.255.0 | 192.168.10.1 |
| PC6 | SW3 Fa0/3 | 20 Support | 192.168.20.13 | 255.255.255.0 | 192.168.20.1 |

| Link | Port A | Port B |
| --- | --- | --- |
| SW1 to R1 (trunk) | SW1 Gi0/1 | R1 Gi0/0 |
| SW1 to SW2 (trunk) | SW1 Fa0/1 | SW2 Fa0/1 |
| SW1 to SW3 (trunk) | SW1 Fa0/2 | SW3 Fa0/1 |

# 3. Task 1: Create VLANs and Assign PC Ports

Create the VLANs on all three switches (shown for SW1; repeat on SW2 and SW3):

Switch> enable

Switch# configure terminal

Switch(config)# hostname SW1

SW1(config)# vlan 10

SW1(config-vlan)# name Sales

SW1(config-vlan)# vlan 20

SW1(config-vlan)# name Support

SW1(config-vlan)# exit

Assign the PC ports as access ports. SW1 (Fa0/3 Sales, Fa0/4 Support):

SW1(config)# interface fastEthernet 0/3

SW1(config-if)# switchport mode access

SW1(config-if)# switchport access vlan 10

SW1(config-if)# interface fastEthernet 0/4

SW1(config-if)# switchport mode access

SW1(config-if)# switchport access vlan 20

SW1(config-if)# end

SW2 and SW3 (Fa0/2 Sales, Fa0/3 Support):

SW2(config)# interface fastEthernet 0/2

SW2(config-if)# switchport mode access

SW2(config-if)# switchport access vlan 10

SW2(config-if)# interface fastEthernet 0/3

SW2(config-if)# switchport mode access

SW2(config-if)# switchport access vlan 20

SW2(config-if)# end

Verify with show vlan brief on each switch.

# 4. Task 2: Configure and Verify Trunks

SW1 has three trunk ports (to SW2, SW3 and the router):

SW1(config)# interface range fastEthernet 0/1 - 2

SW1(config-if-range)# switchport mode trunk

SW1(config-if-range)# exit

SW1(config)# interface gigabitEthernet 0/1

SW1(config-if)# switchport mode trunk

SW1(config-if)# end

SW2 and SW3 each have one trunk port toward SW1:

SW2(config)# interface fastEthernet 0/1

SW2(config-if)# switchport mode trunk

SW2(config-if)# end

Verify with show interfaces trunk. Expected output on SW1:

SW1# show interfaces trunk

Port        Mode         Encapsulation  Status        Native vlan

Fa0/1        on           802.1q         trunking      1

Fa0/2        on           802.1q         trunking      1

Gig0/1       on           802.1q         trunking      1

Port        Vlans allowed on trunk

Fa0/1        1-1005

Fa0/2        1-1005

Gig0/1       1-1005

Port        Vlans allowed and active in management domain

Fa0/1        1,10,20

Fa0/2        1,10,20

Gig0/1       1,10,20

Screenshot to submit: the show interfaces trunk output from SW1 (and SW2/SW3 if possible) taken in Packet Tracer. Both VLAN 10 and 20 should appear in the allowed and active list.

Check: a Sales PC on one switch can ping a Sales PC on another switch (for example PC1 to PC3), which proves the trunks work. Support PCs likewise (PC2 to PC4).

# 5. Task 3: Router-on-a-Stick

Router> enable

Router# configure terminal

Router(config)# hostname R1

R1(config)# interface gigabitEthernet 0/0

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface gigabitEthernet 0/0.10

R1(config-subif)# encapsulation dot1Q 10

R1(config-subif)# ip address 192.168.10.1 255.255.255.0

R1(config-subif)# exit

R1(config)# interface gigabitEthernet 0/0.20

R1(config-subif)# encapsulation dot1Q 20

R1(config-subif)# ip address 192.168.20.1 255.255.255.0

R1(config-subif)# end

R1# copy running-config startup-config

Verify with show ip interface brief (Gi0/0, Gi0/0.10 and Gi0/0.20 up/up) and show ip route (two connected routes, 192.168.10.0/24 and 192.168.20.0/24).

Alternative: a multilayer switch can do the same with ip routing and an interface vlan 10 / interface vlan 20 SVI for each VLAN, with no router needed.

# 6. Task 4: Connectivity Tests and Troubleshooting

| Test | Expected | Result (fill in) |
| --- | --- | --- |
| PC1 to 192.168.10.1 (gateway) | Success |  |
| PC1 to PC3 (Sales, different switch) | Success |  |
| PC2 to PC4 (Support, different switch) | Success |  |
| PC1 to PC2 (Sales to Support, same switch) | Success |  |
| PC1 to PC6 (Sales to Support, different switch) | Success |  |
| PC5 to PC4 (Sales to Support, different switch) | Success |  |

The first ping may time out while ARP completes; repeat it before deciding that it failed.

## Troubleshooting guide

| Problem | How to check | Fix |
| --- | --- | --- |
| Wrong default gateway on a PC | ipconfig on the PC | Set the gateway to 192.168.10.1 or 192.168.20.1 |
| Wrong sub-interface IP or mask | show ip interface brief on R1 | Correct the ip address command |
| Wrong VLAN number in encapsulation | show running-config on R1 | Use encapsulation dot1Q 10 / 20 |
| Port to router is not a trunk | show interfaces trunk on SW1 | switchport mode trunk on Gi0/1 |
| VLAN not allowed or missing on a trunk | show interfaces trunk, show vlan brief | Create the VLAN; switchport trunk allowed vlan add 20 |
| PC port in the wrong VLAN | show vlan brief | switchport access vlan 10 or 20 |
| Router interface shut down | show ip interface brief | no shutdown on Gi0/0 |

Example fix recorded in this practical: pings from Sales to Support failed because the PC gateway was set wrongly. After correcting the gateway to the sub-interface IP, pings between VLAN 10 and VLAN 20 succeeded. (Replace with the fault you actually found.)

# 7. Conclusion

Three switches were connected with 802.1Q trunks, and PCs were assigned to VLAN 10 (Sales) and VLAN 20 (Support). Router-on-a-stick sub-interfaces provided inter-VLAN routing. Verification with show interfaces trunk and ping tests confirmed that VLAN traffic crosses the switches and that Sales and Support can communicate through the router.

# 8. Submission Checklist

- Topology screenshot with VLAN and IP labels

- show interfaces trunk screenshot

- show vlan brief and show ip route screenshots

- Ping results between VLAN 10 and VLAN 20

- Notes on any problem found and how it was fixed